These quality designs are created and copyright protected by Erina and distributed through www.oregonpatchworks.com.  You can find these and more beautiful designs at

http://www.oregonpatchworks.com/collections.php?id=DC

The OPW Mall – Where your dreams become a reality, one stitch at a time.

Feel free to contact us at any time and visit us again soon!

support@oregonpatchworks.com

erinasdesigns@gmail.com

THREAD – MARATHON RAYON - Use a light color for the FSL background and a dark color for the designs on top of the bookmark so that the contrast is clearly visible.

Mylar FSL Christmas Bookmark

Hoop a double layer of Vilene in a 5x7 hoop

The bookmark has 4 color changes, but for Color change 1-3 the same thread must be used and for Color change 4 a contrasting color :

- FSL Background, use a very light color

When the machine stops after this color the Mylar can be inserted.

I used an Opal color.

- Tacking stitches for Mylar. Use the same thread as for Color 1.

Remove hoop from machine but do not un-hoop.

Cut or tear the Mylar (depending on which Mylar you are using) around the edges

Place hoop back into the machine.

- The pattern that will hold the Mylar in place will stitch now. Use the same thread as for Color 1.

- Christmas design on top of background will stitch now as well as the satin stitch outline. Use a contrasting thread that will stand out clearly against Color change 1-3.

Instructions for Freestanding Lace:

1.  Stitch out on Vilene Avlon Plus or any other water dissolvable stabilizer.  A piece of Avlon on top will prevent the designs from sinking into the fabric

2.  Take out of the hoop and cut out around edge when the design is finished .

3.  Rinse design in boiling hot water and leave a short while in the boiling hot water (about 30 seconds).  Rinse design under cold tapwater for 30 seconds. Leave the gift tag in the hot water for longer if you want a softer effect.

4.  If necessary the design can be lightly pressed with a dry cloth and medium (not too hot) iron on the wrong side of the design. The designs must be laid out on  a soft towel.

5. Put design between two layers of  soft but firm cloth and a weight on top. After a few hours take out and leave to dry completely.

Enjoy the designs

Erina